Homework 4 Answer Sheet

Please state the name, SID and email of each member of your group.

| member | name | SID | email |
| --- | --- | --- | --- |
| #1 (contact person) | Weikai Chen | 40159549 | weikachen4-c@my.cityu.edu.hk |
| #2 | NG KA HUNG | 58533046 | kahungng9-c@my.cityu.edu.hk |
| #3 | Chan Chi Him | 58542344 | chchan2453-c@my.cityu.edu.hk |

Logisim Version: Logisim-win-2.7.1

OS (Window/MAC/Linux): Window

- Do all members make significant contributions to this homework? If not, please specify the details.

All members are distributed equally.

- Please explain how many types of instructions are supported in your processor, and explain the format of each type of instructions (e.g., which bits are used as the operation or function code, which bits are used to index the 1st, 2nd or 3rd operand, and which bits are used to store the immediate number). You can draw figures to better explain your answer.

The sum of the numbers

x = 58533046 + 58542344 + 40159549

x = 157234939

x mod 7 = 1

In our processor, we used in total three types of instructions:

R-type for: add, and, or, move

For R-type

| Bits place | functions |
| --- | --- |
| 1-3 | Write (destination) register |
| 4-6 | Read register 1 |
| 7-9 | Read register 2 |
| 12-15 | Opcode |

I-type for: li, addi, andi, ori, ble, bne, load, store, halt

For I-type

| Bits place | functions |
| --- | --- |
| 0-3 | immediate |
| 4-9 | Read register |
| 4-6 | Write register |
| 12-15 | Opcode |

J-type for: jump, call, rtn

For J-type

| Bits place | functions |
| --- | --- |
| 0-11 | Address |
| 12-15 | Opcode |

- Please explain the format of each instruction (including the format of this instruction and its operation codes, and other information if needed).

| li | Instruction type: I-type Opcode: 1001 (li has same opcode to addi, but the $rs is setted to zero) format: - opcode (4 bits) - rt(6bits) - Immediate (4 bits) |
| --- | --- |
| add | Instruction type: R-type Opcode: 0000 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - rd(3bits) |
| and | Instruction type: R-type Opcode: 0010 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - rd(3bits) |
| or | Instruction type: R-type Opcode: 0011 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - rd(3bits) |
| load | Instruction type: I-type Opcode: 0101 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (6bits) |
| store | Instruction type: I-type Opcode: 0110 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (6bits) |
| move | Instruction type: R-type Opcode: 0001 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - rd(3bits) |
| addi | Instruction type: I-type Opcode: 1001 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (6bits) |
| andi | Instruction type: I-type Opcode: 1010 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (6bits) |
| ori | Instruction type: I-type Opcode: 1011 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (6bits) |
| ble | Instruction type: I-type Opcode: 1000 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (4 bits) |
| bne | Instruction type: I-type Opcode: 1101 format: - opcode (4 bits) - rs (3bits) - rt(3bits) - Immediate (6bits) |
| jump | Instruction type: J-type Opcode: 1111 format: - opcode (4 bits) - Address (12 bits) |
| call | Instruction type: J-type Opcode: 0100 format: - opcode (4 bits) - Address (12 bits) |
| rtn | Instruction type: J-type Opcode: 1110 format: - opcode (4 bits) - Address (12 bits) |
| halt | Instruction type: I-type Opcode: 1100 format: - opcode (4 bits) |

- Fill the following tables with the machine codes of each instruction of the testing programs:

Test program 1:

| instruction | machine code (binary) | machine code (hex) |
| --- | --- | --- |
| li $r1, 1 | 1001 0000 0000 0001 | 0x9001 |
| li $r2, 2 | 1001 0000 0001 0010 | 0x9012 |
| li $r3, 10 | 1001 0000 0010 1010 | 0x902A |
| add $r2, $r1, $r2 | 0000 0000 1000 0010 | 0x0082 |
| ble $r2, $r3, -1 | 1000 0001 0001 0010 | 0x8112 |
| halt | 1100 0000 0000 0000 | 0xC000 |

Test program 2:

| instruction | machine code (binary) | machine code (hex) |
| --- | --- | --- |
| li $r1, 6 | 1001 0000 0000 0110 | 0x9006 |
| li $r2, 5 | 1001 0000 0001 0101 | 0x9015 |
| andi $r3, $r1, 3 | 1010 0001 0000 0011 | 0xA103 |
| ori $r4, $r3, 8 | 1011 0001 1010 1000 | 0xB1A8 |
| halt | 1100 0000 0000 0000 | 0xC000 |

Test program 3:

| instruction | machine code (binary) | machine code (hex) |
| --- | --- | --- |
| li $r1, 6 | 1001 0000 0000 0110 | 0x9006 |
| li $r2, 5 | 1001 0000 0001 0101 | 0x9015 |
| and $r3, $r1, $r2 | 0010 00 001 000 010 0 | 0x2084 |
| li $r8, 0 | 1001 0000 0111 0000 | 0x9070 |
| store $r3, $r8 | 0110 00 000 111 010 0 | 0x6074 |
| or $r4, $r1, $r2 | 0011 00 001 000 011 0 | 0x3086 |
| li $r8, 1 | 1001 0000 0111 0001 | 0x9071 |
| store $r4, $r8 | 0110 00 000 111 011 0 | 0x6076 |
| li $r8, 1 | 1001 0000 0111 0001 | 0x9071 |
| load $r7, $r8 | 0101 00000 111 110 0 | 0x507C |
| halt | 1100 000 000 000000 | 0xC000 |

Test program 4:

| instruction | machine code (binary) | machine code (hex) |
| --- | --- | --- |
| li $r1, 6 | 1001 0000 0000 0110 | 0x9006 |
| li $r2, 4 | 1001 0000 0001 0100 | 0x9014 |
| call 7 | 0100 000000000110 | 0x4006 |
| move $r4, $r3 | 0001 00 010 011 0000 | 0x1130 |
| li $r1, 7 | 1001 00 000 000 0111 | 0x9207 |
| li $r2, 8 | 1001 00 110 001 1000 | 0x9318 |
| call 3 | 0100 000000000010 | 0x4002 |
| move $r5, $r3 | 0001 00 010 100 0000 | 0x1140 |
| jump 3 | 1111 000000000010 | 0xF002 |
| add $r3, $r1, $r2 | 0000 00 001 000 010 0 | 0x0084 |
| rtn | 1110 000000000000 | 0xE000 |
| halt | 1100 000 000 000000 | 0xC000 |

Appendix(How to open our circuit files correctly):

- Unzip the CPU_Components.zip

- Open the CPU.circ then import each components in the folder according to the instructions

- Open memory files for testing